# Experiment 2: Diesel Engine’s Thermal Efficiency

The overall efficiency of an engine to convert the energy contained in the fuel to mechanical work and consequently into electrical power in a generator set depends on the following efficiencies:

- Combustion efficiency

- Thermal efficiency

- Volumetric efficiency

- Mechanical efficiency

In brief, combustion efficiency refers to the efficiency of the combustion process to release the heating content of the fuel; the thermal efficiency refers to the efficiency of the thermal cycle to convert thermal power to mechanical power; the volumetric efficiency refers to the engine’s ability to fill the displacement volume fully by air (air mass affect produced power); and the mechanical efficiency takes into account various friction losses in the engine. A diesel generator set would also have a generator efficiency which refers to the conversion efficiency of the mechanical power to the electrical power.

The objective of this experiment is to compare the theoretical thermal efficiency of a diesel engine with experimentally measured efficiency. The following typical values are assumed for the engine (comment on validity of these assumptions in your lab report):

- Combustion efficiency – Assumed 90-95%

- Thermal efficiency   --- Calculated

- Volumetric efficiency – Assumed 80-85%

- Mechanical efficiency – Assumed 85-90%

Carefully review the provided handouts and safety precautions before starting the experiment.

Figure 1P-v diagram for diesel engine.

Theory

The thermal cycle for a diesel engine is shown in Figure 1. Thermal efficiency of a diesel engine may be approximated based on the engine’s compression ratio, rc , specific heat ratio of air, k=1.4, and the cut-off ratio, b, in the diesel cycle:

(ηt)DIESEL=wnetqin=1-1rck-1βk-1k(β-1)           (Eq. 1)

The brake power measure (i.e., the net engine power measured) is the result of all previously mentioned efficiencies:

Wb=mfHVηcηtηvηm        (Eq. 2)

The compression ratio of the diesel engine is 22 and a cut-off ratio of 2 can be assumed for this lab.

Engine Specifications:

Fuel Specifications:

# Data Sheet

Obtain at least 6 data points over the following range:

Speed: 1500-3000 rpm

Torque: 9.5-10 N.m

Engine power: 1.5-3kW